PŘÍLOHA – seznam požadovaného lékařského vybavení

- Mobile surgical light (1 pc)

A mobile surgical light designed for small operations.

Technical parameters: Light intensity at 0.5m distance 110,000 lux Working distance 70-140 cm Fixed focus Stand model 26 LEDs

- Examination and surgical chair (1 pc)

Fully equipped examination chair for day surgery, recovery or diagnostic procedures.

Technical parameters: total rest area 2 123 mm, safe working load 205 kg, maximum weight 190 kg, Back section 70°, armrest vertical position 42° / - 8.5°, armrest horizontal position 360°

- Suction pump (1 pc)

2 l suction cup with a press-on lid, a suction cup holder, a PVC suction circuit with a shut-off valve and a mobile stand.

Technical parameters:

- Power supply 230 V, 50/60 Hz, 110 VA

- Vacuum level – High vacuum -92 kPa (-690 mmHg)

- Free air flow 40 l/min

- Patient monitor (1 pc)

Portable patient monitor for monitoring the patient's basic vital functions; color display.

- Stretcher (1 pc)

- External dimensions: 211 x 76 (86) cm

- Mattress platform: 193 x 66 cm

- Height adjustment: 56 – 89 cm

- TR/ATR Tilt: 18°/18°

- Safe working load: 320 kg

- Weight with basic equipment: 120-135 kg

- Hot air sterilizer (1 pc)

For use in the healthcare sector for the hot air / dry heat sterilization process of both unpackaged and packaged medical devices, including invasive devices intended by their manufacturers for hot air / dry heat sterilization.

- Technical parameters:

- Volume 22 l

- Power supply 230V, 50/60 Hz

- Max. power consumption 0.96 kW

- Working temperature 10°C above ambient temperature up to 250°C

- Stainless steel 1.4301 (AISI 304)

- Syringe pump (1 pc)

- Syringe 10ml: 0.1-300ml/h

- Syringe 20ml: 0.1-400ml/h

- Syringe 30ml: 0.1-500ml/h

- Syringe 50 ml: 0.1-1200 ml/h

- Flow error margin: ±3%

- Limit infusion volume range: 0.1~999.9ml (infusion interval 0.1ml)

- Total estimated volume range: 0.1~999.9ml (infusion interval 0.1ml)

- K.V.O rate (vein maintenance function): 1 ~ 10 ml/h adjustable

- Fast flow (Purge): 0.1 ~ 1200 ml/h adjustable

- Bolus 0.1 ~ 1200 ml/h, 0.1 ~ 1000 ml adjustable

- Nurse call DC12V, 1A

- Doctor’s chair (1 pc): seat height 45-60 cm

- Infusion stand (1 pc): height ≥ 160cm

- Instrument table (1 pc): height x width x depth 80 x 60 x 60 cm

- Stainless steel trolley (1 pc): height max. 80 cm

- Pharmacy cabinet (1 pc): height cca. 200 cm, width min. 60 cm

- Mobile ultrasound unit (1 pc)

For abdominal, cardiological, OB/GYN, radiological, urological uses

- Probe types: linear array (multi frequency), convex array (multi frequency), endovaginal, endorectal, dedicated 3-D/4-D, hockey stick, phased array

- Scan modes: M-mode, B –mode(2-D), 3-D (automatic), 4-D (automatic), harmonic

- Doppler: CDI, PDI, 3-D/4-D, continuous wave, pulsed wave, duplex / triplex mode

- Image Display and Processing: automated image optimization, gray scale level ≥ 256, image magnification, monitor ≥ 12 in), maximum display ≥ 240 mm, pre-/post-processing, real-time image, frozen image, split screen

- Analysis packages: Cardiac, vascular, Auto IMT (RF in real time), X Strain 2-D and 4-D strain/strain rate analysis, stress echo, abdominal, neck ultrasound, adult cephalic (TCD), breast, rheumatology, neonatal, pediatric, OB/GYN, small parts, urology

- Medical-approved power supply board, 220/240 VAC, 50/60 Hz

- Near infra scanner (1 pc)

- Handheld near-infrared (NIR) device used for rapid screening of intracranial hematomas

- Eye-safe NIR diode laser as the illumination source

- Optical detector captures reflected light for analysis

- Onboard embedded computer performs real-time signal processing

- Detects supratentorial hematomas (above the tentorium)

- Sensitivity threshold: hematomas larger than roughly 3.5 mL

- Depth sensitivity: up to about 2.5 cm from the brain surface

- Eight standard scan locations: bilateral frontal, temporal, parietal, and occipital regions

- Electrosurgical unit (1 pc)

Technical parameters: Maximum power 400 W Working frequency 460 kHz Monopolar and bipolar mode Control by pneumatic waterproof footswitches or handswitch

- UVC disinfection Robot (1 pc)

- 254 nm UVC wavelength — the germicidal band used for surface and air disinfection

- High-intensity UV output — designed for rapid microbial inactivation

- 360-degree coverage — lamps arranged for full-room irradiation

- Effective disinfection radius of several meters — suitable for medium-sized rooms

- >99.99% pathogen reduction — effective against viruses, bacteria, spores, and fungi